# PRoject Description

# Vehicle Trajectory Prediction For Various Types Of Road

Thanh Hung Duong, Yunpeng Zhang

Abstract

- General purposes: What is vehicle trajectory prediction (VTP), and why is it important? Why do we need to predict vehicle trajectories on different road types?

- Specific goals: Developing various artificial neural networks (ANN) models of VTP on specific road types.

- Research design: 3 steps:

  - Autonomous extraction of vehicle trajectory data from public traffic cameras

  - Development of ANN models for VTP on each road types

  - Application of Big Data pipeline for both two previous steps

- Methods:

  - Focus on five types of road: straight, curved, highway, intersection, and roundabout

  - Use R-CNN to recognize vehicle → time series analysis for vehicle tracking → post-processing to compute related parameters such as location, speed, acceleration, lane,…

  - Apply relational recurrent neural networks (RRNNs) and Long Short-Term Memory (LSTM) for VTP

  - Use Amazon Elastic compute cloud (Amazon EC2) to improve the autonomous extraction and predictions

- Significance:

  - Vehicle motions on each road type have a unique set of characteristics. This project will help improve the movements and lane changing decisions of autonomous vehicles at any specific road type

  - This project shows an example of extracting useful information from traffic cameras, whose primary functions are to measure traffic flow and to determine traffic light timing. Compared to the traditional way of collecting this kind of data, which uses a car that equipped with cameras and a GPS tracking device, our approach is much less expensive.

  - With the help of Amazon EC2, autonomous vehicles can simulate the best trajectory at any road in real-time

# research background

As autonomous vehicles (AV) offer improvements in traffic safety, life quality, and supply chain management, more and more companies have participated in developing them. When the regulations for governing automated driving in California went into effect for the first time in 2014, there were only three manufacturers that got permits, namely Volkswagen, Mercedes, and Google. (Matt Williams 2014) So far, in July 2020, that number went up to 66, in which Waymo LLC and Nuro Inc are allowed to test their autonomous vehicle without a driver. (Autonomous Vehicle Testing Permit Holders - California DMV n.d.) Moreover, autonomous cars got strongly supported by many governments around the world, such as the United States, the Netherlands, Singapore, and Norway. (KPMG 2019; National Science and Technology Council and US DOT 2020)

For safe and efficient navigation, autonomous vehicles should be able to mimic the ability to analyze and forecast the behavior of surrounding vehicles like an experienced human driver.  This ability is crucial when making tactical driving decisions like lane changing, overtaking. Since it is significantly associated with other driver's manners and road types, vehicle trajectory prediction (VTP) is a challenging but stimulating task. Figure 1 shows the number of articles that contain the term "Vehicle trajectory prediction" from 2000 to 2019; the data was collected from Google Scholar. The number of studies involved in the prediction of vehicle trajectories has increased over the year, and most of them are within the last five years.

Figure 1. Numbers of Google scholar results with the term "Vehicle trajectory prediction" between 2000 and 2019

Generally, the VTP models can forecast vehicle trajectories in the next few seconds based on the historical path of the object vehicle during the last few seconds. Those models could be categorized as maneuver-based models and end-to-end models, as illustrated in Figure 2 (Dai, Li, and Li 2019) (Dai, Li, and Li 2019). The main difference between them is that the maneuver-based models have one extra step: maneuver recognition, which can indicate the motion maneuver. Recently, while approaches for maneuver-based models have shifted to the traditional Recurrent neural networks (RNN) models (Patel et al. 2018; Xing, Lv, and Cao 2020), those of end-to-end models have developed based on the Long short-term memory (LSTM) model (Altché and Fortelle 2018; Deo and Trivedi 2018b; Park et al. 2018). RNN is a feed-forward artificial neural network that can use their internal state to process variable-length sequences of inputs. In comparison, the LSTM is a class of RNN that can overcome the vanishing gradient problem. Both have proven their merits in classifying, predicting based on time series data.

Figure 2. Two typical types of vehicle trajectory prediction model

In most of the VTP models, the motion maneuvers were set as straight highway roads for the sake of simplification, and those models thus were developed based on the data sets from freeways (I-80 and US-101) of the Next Generation Simulation (NGSIM) project. In each case, the data set includes complete vehicle trajectories for all vehicles over the freeway from the entrance to exit. Although many VTP studies used the NGSIM data, an adequate number of researchers have found the unrealistic and inaccuracy in them. Moreover, the data sets were collected more than ten years ago, and they contain the trajectories of a few thousand vehicles over a short distance and duration. Therefore, the models that built on it may produce senseless predictions.

# research problem

In summary, the research problems in previous studies on vehicle trajectory predictions are

- They focused on developing the prediction algorithm but not the influence of contextual features such as road structure from the scene that may affect the vehicle's future trajectory. The prediction models in each study were trained on a single road type; there thus are no prediction models for multiple kinds of road structures. It is vital and urgent to develop those models as transportation infrastructures in modern cities are complex, and many roads are mixtures of more than two road structure types.

- Not only the number of decent public vehicle trajectory data sets is scarce, but also most of the available data sets were collected from a simple road structure like NGSIM.

- In order to have the ability to perform well in a wide range of situations, the prediction model is needed to be trained on the data set collected from as many as possible road types. In real life, the road structures are diverse; hence that kind of data set can be considered big data. However, so far, the implementations of Big Data in the previous prediction model were not mentioned or were brought up limitedly.

In this project, we aim to develop prediction models for vehicle trajectories on five main road types: straight, curved, highway, intersection, and roundabout via LSTM Neural Networks. As available public data sets mostly are collected from the highway, we also plan to develop a novel algorithm to extract vehicle trajectories from other road types via traffic cameras or drones. The algorithms like R-CNN, YOLO will be applied to find vehicles in video frames and then track their movements. Moreover, to able to predict in real-time, techniques for Big Data will be implemented in both developments via High Performance Computing

# literature review

## Vehicle trajectory prediction

The environment of autonomous vehicles contains both static and dynamic obstacles. While static hindrances like work zone, pavement, shoulders can be detected and be dodged easily, it is challenging to prevent a collision with dynamic objects as their movements usually are unknown or unpredictable. Compared to slow dynamic obstacles like pedestrians, surrounding vehicles are much more dangerous since their speeds are relatively high, and their motions depend on a vast number of factors. Therefore, to ensure safety for the operation of autonomous vehicles, researchers have concentrated on the prediction of the future trajectories of the surrounding cars.

### A review of techniques to predict vehicle trajectory

| Control Strategy | Strength(s) | Weakness |
| --- | --- | --- |
| Geometric & Kinematic | - Adequate performance (experimentally validated) in conditions without disturbances (e.g., wind, road banking) - Excellent tracking performance and robustness at moderate speeds (e.g., kinematic) | - Do not consider vehicle dynamics - Steady-state error increases for high-speed driving (e.g., geometric) - Unsuitable for high-speed driving as dynamics are neglected (e.g., kinematic) - Requires smooth and continuous reference trajectories |
| Classical | Established method with good performance for non-linear systems Robust closed-loop performance against uncertainties and noise (e.g., SMC) | Tuning of controller parameters is tricky (e.g., PID) Robust performance only in limited scenarios (e.g., SMC) Control law is sensitive to path curvature variations (e.g., SMC) |
| Dynamic state feedback | Consider vehicle dynamics in calculating control law Optimization shifted offline resulting in simple implementation of control law | Obtaining vehicle states (e.g., wheel forces, slip angles, torques etc.) is non-trivial Control law is sensitive to path curvature variations (e.g., LQR) |
| Neural Network | Sufficient training can make the behavior very human-like to make the automated car feel natural | Controller tuning requires simulation with large amounts of real world (training) data No failure explanations possible |
| Fuzzy Logic | Closed-loop system acts similar to a human-driver (because of human-like rules) | Controller tuning is not systematic with no formal stability analysis Rules can become unmanageable if number of variables is large |
| Model Predictive Control (MPC) | Systematic design procedure Ability to include system and actuator constraints in design procedure Inclusion of vehicle and tire dynamics in control problem | Non-linear MPCs with have high computing requirements making them unsuitable for high-speed driving environments The tracking performance is sensitive to the accuracy of prediction model Larger tuning parameter set compared to industry standard PID |

Zhu et al. claimed that there are three critical challenges in predicting vehicle trajectories:(Zhu et al. 2019)

- Learning sufficient representations that consider mutual interaction information among all related surrounding vehicles;

- Predicting trajectories with the uncertainty which is functionally characterized by explicit distributions

- Developing a generalized model that can summarize and learn the underlying knowledge from limited size of data, thus can adapt to new scenarios efficiently.

### Recurrent Neural Networks for vehicle trajectory prediction

In 2018, Zyner et al., presented a model that produces a multi-modal path prediction with uncertainty via Recurrent neural network, and is clustered into a meaningful output. (Zyner, Worrall, and Nebot 2018) Highlights of their research are:

- Real-world, real-time - Unlike many works that use the NGSIM dataset, we do not filter the input data at all, and so all data taken at time t does not rely on data taken at time t++ for filtering or analysis. The data was collected with a Lidar enabled vehicle as opposed to an overhead camera, and so it includes all data tracking noise, and mimicking the perspective issues of a vehicle driving through the scene.

- Minimal map knowledge - While it is necessary for the vehicle to know it is approaching a roundabout, it does not need high definition maps for the algorithm to work.

- Generalizable across intersections - We demonstrate that this model generalizes between similar-sized junctions by using a dataset that spans multiple intersections.

- Large, naturalistic dataset - We present a dataset that accumulates over 60 hours of vehicle data of naturalistic human drivers traversing five different roundabout style intersections in Sydney. To the author's knowledge, this is the largest dataset of unsignalized intersections publicly available, and can be downloaded at: http://its.acfr.usyd.edu.au/datasets/

Recently, the team of Messaoud developed a relational recurrent neural network (RRNNs) based encoder-decoder architecture where the encoder analyzes the patterns underlying in the past trajectories, and the decoder generates the future trajectory sequence. (Messaoud et al. 2019) It combines the advantages of LSTMs in sequence modeling and the power of attention mechanism to capture the spatial intervehicles interactions. It is characterized by:

- Per block information storing: Input information is selectively stored into separate interacting blocks based on their content.

- Relational reasoning: Some vehicles are more likely to be related to or influenced by the other vehicles because of some features.

- No distance constrained analysis: Dependence between vehicles is not always tied to proximity in space.

- Different focusing: Different relations are encoded based on selective attention to a set of input information.

They used the new publicly available naturalistic vehicle trajectory highD dataset to train and validate their model in the task of trajectory prediction. When compared to LSTM based encoder decode model, their model provided better results in terms of longitudinal and lateral prediction RMSE.

In the same year, recurrent neural networks (RNNs) were incorporated with neural process (NP) to create new model: attentive recurrent neural process (ARNP).(Zhu et al. 2019) By assuming that the observed trajectories are generated from multiple realizations of a stochastic process, their proposed model first comprehensively captures the sequential interaction behavior through a RNN framework. Then the predictive trajectory is obtained by estimating a distribution of the mapping from surrounding vehicle information to corresponding target vehicle trajectory conditioned on the history input-output pairs. Their proposed model effectively capture the sequential information and the global underlying property of a traffic scenario. In a real-world trajectory prediction task, ARNP outperforms a number previous approaches in the aspect of prediction accuracy, variance, and probability expressiveness by predicting a explicit distribution.

### Long Short-Term Memory neural networks for vehicle trajectory prediction

#### Overview

There are 83 papers that contains both two terms: "vehicle trajectory prediction" and "long short-term memory" according to Google scholar. Many of them come from ArXiv journal. The below table shows some of the article that use LSTM for vehicle trajectory prediction.

| Index | Paper Name | First author | Year | Citation | Average citation per year |
| --- | --- | --- | --- | --- | --- |
| 1 | Convolutional Social Pooling for Vehicle Trajectory Prediction | Nachiket Deo | 2018 | 110 | 36.7 |
| 2 | An LSTM network for highway trajectory prediction | Florent Altché | 2017 | 129 | 32.3 |
| 3 | Sequence-to-Sequence Prediction of Vehicle Trajectory via LSTM Encoder-Decoder Architecture | Seong Hyeon Park | 2018 | 96 | 32.0 |
| 4 | Probabilistic vehicle trajectory prediction over occupancy grid map via recurrent neural network | ByeoungDo Kim | 2017 | 122 | 30.5 |
| 5 | Multi-Modal Trajectory Prediction of Surrounding Vehicles with Maneuver based LSTMs | Nachiket Deo | 2018 | 84 | 28.0 |
| 6 | TraPHic: Trajectory Prediction in Dense and Heterogeneous Traffic Using Weighted Interactions | Rohan Chandra | 2019 | 44 | 22.0 |
| 7 | Modeling vehicle interactions via modified LSTM models for trajectory prediction | Shengzhe Dai | 2019 | 19 | 9.5 |
| 8 | Intention-aware Long Horizon Trajectory Prediction of Surrounding Vehicles using Dual LSTM Networks | Long Xin | 2018 | 20 | 6.7 |
| 9 | Online Vehicle Trajectory Prediction using Policy Anticipation Network and optimization-based Context Reasoning | Wenchao Ding | 2019 | 9 | 4.5 |
| 10 | Personalized Vehicle Trajectory Prediction Based on Joint Time-Series Modeling for Connected Vehicles | Yang Xing | 2020 | 2 | 2.0 |
| 11 | See the Future: A Semantic Segmentation Network Predicting Ego-Vehicle Trajectory With a Single Monocular Camera | Yuxiang Sun | 2020 | 2 | 2.0 |
| 12 | GISNet: Graph-Based Information Sharing Network For Vehicle Trajectory Prediction | Ziyi Zhao | 2020 | 1 | 1.0 |
| 13 | Multi-modal vehicle trajectory prediction based on mutual information | Cong Fei | 2020 | 0 | 0.0 |
| 14 | Vehicle trajectory prediction using recurrent LSTM neural networks | ALEXANDER BÜKK | 2020 | 0 | 0.0 |
| 15 | A Multi-Modal States based Vehicle Descriptor and Dilated Convolutional Social Pooling for Vehicle Trajectory Prediction | Huimin Zhang | 2020 | 0 | 0.0 |
| 16 | Interaction-Aware Trajectory Prediction of Connected Vehicles using CNN-LSTM Networks | Xiaoyu Mo | 2020 | 0 | 0.0 |
| 17 | A Dual Learning Model for Vehicle Trajectory Prediction | Mahrokh Khakzar | 2020 | 0 | 0.0 |

To save time, we will review the first three papers with highest citations to understand the basic and 2 random papers in 2020 to get the current trend

#### Convolutional Social Pooling for Vehicle Trajectory Prediction (Deo and Trivedi 2018a)

Prediction of future motion of surrounding vehicles is a challenging problem because:

- Vehicle trajectories tend to be highly non-linear over longer time horizons due to decisions made by the driver.

- Driver behavior tends to be inherently multi-modal, where a driver could make one of many decisions under the same traffic circumstances.

- Interaction between vehicles tends to affect their motion. The large number of possible configurations of all vehicles in a scene can make this difficult to model.

Despite these challenges, there is structure to vehicle motion that can be exploited:

- Vehicle motion can be binned into maneuvers, which can account for the multi-modal nature of future motion. For example, a vehicle approaching its leading vehicle at a much faster speed would either brake and slow down or change lane to overtake.

- There is a well-defined lane structure and a direction of motion on freeways. This can be exploited to model interaction between vehicles

Authors proposed an LSTM encoder-decoder based model for vehicle motion prediction for the case of freeway traffic.

- Convolutional social pooling: We propose a novel social pooling layer as an alternative to that proposed in [1]. We apply convolutional and max-pooling layers instead of a fully connected layer to social-tensors of LSTM states that encode the past motion of neighboring vehicles.

- Maneuver based decoder: Our LSTM decoder generates the probability distribution over future motion for six maneuver classes and assigns a probability to each maneuver class. This accounts for the multi-modal nature of vehicle motion.

The proposed model outperforms the reported state of the art on two large publicly available datasets of vehicle trajectories. Its outputs multi-modal distributions for future motion of vehicles based on maneuver classes.

One limitation of the current approach is that it relies purely on vehicle tracks to infer maneuver classes and future trajectories. A lot of complementary information can be captured using visual and map based cues. These could be used to improve the accuracy of maneuver classification and thus, that of future motion prediction.

#### An LSTM Network for Highway Trajectory Prediction (Altché and Fortelle 2018)

LSTM are able to keep a memory of previous inputs, LSTMs are considered particularly efficient for time series prediction [21] and have been widely used in the past few years for pedestrian trajectory prediction [16], [17] or to predict vehicle destinations at an intersection [11], [22].

Their main contribution is the design of an LSTM network to predict car trajectories on highways, which is notably critical for safe autonomous overtaking or lane changes, and for which truly little literature exists.

- A particular challenge for this problem is that highway driving usually comprises a lot of constant velocity phases with rare punctual events such as lane changes, which are therefore hard to learn correctly. For this reason, many authors rely on purposely recorded [7] or handpicked [23], [24] trajectory sets which are not representative of actual, average driving. Therefore, the real-world performance of trained models can be significantly different.

- A second contribution of this article is that we train and validate our model using the entire NGSIM US101 dataset [25] without a-priori selection, and show that we can predict future trajectories with a satisfying average RMS error below 0.7m (laterally) and 2.5ms-1 (longitudinally) when predicting 10s ahead. To the best of their knowledge, no other published learning technique was demonstrated with similar results using a dataset representative of real-world conditions.

#### Sequence-to-Sequence Prediction of Vehicle Trajectory via LSTM Encoder-Decoder Architecture (Park et al. 2018)

Although the reported LSTM based vehicle trajectory models are effective for their own tasks, they either cannot generate the full trajectory sequence or do not provide a simple probabilistic framework that does not require extra components such as CVAE.

Author proposed a new vehicle trajectory analysis and prediction technique to generate the future trajectory of surrounding vehicles given the sequence of the latest sensor measurements. The proposed method is built upon the LSTM encoder-decoder architecture which has shown excellent performance for sequence to sequence tasks [8].

- The LSTM encoder takes the latest trajectory samples for the surrounding vehicles as well as the state information on the ego vehicle and produces the fixed length vector which captures the temporal structure of the past trajectory.

- Based on the fixed length vector, the LSTM decoder generates the future trajectory on the occupancy grid map (OGM). The decoder recursively uses the predicted trajectory sample to generate the subsequent trajectory samples adopting beam search algorithm. With the algorithm, the decoder keeps K locally best sequence candidates in generating the future trajectory sample for each time step [16]. As a result, the proposed model can predict the K most probable hypotheses of the vehicle trajectory under the probabilistic framework.

Their experiment results show that the proposed scheme generates the reasonable trajectory hypotheses of the surrounding vehicles and its prediction accuracy is significantly better than the conventional prediction methods.

#### A Road-Aware Neural Network for Multi-step Vehicle Trajectory Prediction (Cui et al. 2017)

In 2018, the main contributions of the research of Cui et al. are as follows:

– We introduce a novel way of extracting road-aware features for vehicle trajectories. The road-aware features consist of intra-road feature and inter-road feature, which are able to help extract latent patterns accurately from trajectories and lead to a substantial increase in prediction accuracy according to our experiments.

– We propose RA-LSTM to achieve multi-step trajectory prediction. The model leverages LSTM units to capture temporal features from previous trajectory path.

– We conduct extensive experiments to evaluate the performance of proposed model on two real-world trajectory data sets. The experimental results show that our model outperforms three baseline methods by 28%–65% on two datasets.

#### Modeling Vehicle Interactions via Modified LSTM Models for Trajectory Prediction(Dai, Li, and Li 2019)

The long short-term memory (LSTM) model is one of the most commonly used vehicle trajectory predicting models.

Two problems of the existing LSTM models for long-term trajectory prediction in dense traffic:

- Existing LSTM models cannot simultaneously describe the spatial interactions between different vehicles and the temporal relations between the trajectory time series. → the existing models cannot accurately estimate the influence of the interactions in dense traffic.

- Basic LSTM models often suffer from vanishing gradient problem and are, thus, hard to train for long time series.

These two problems sometimes lead to large prediction errors in vehicle trajectory predicting.

They proposed a new model called ST-LSTM with two structural modifications to the basic LSTM models for trajectory prediction:

- The 1st modification: Adopt the idea of Structural-RNN (S-RNN) [25] and model all vehicle trajectories and interactions between vehicles by a new LSTM model. Specially, we construct the temporal relations and spatial interactions as different time series and handle them separately by LSTM models. The new model is more suitable for traffic scenarios than those models (e.g. S-RNN) that have no clear division between temporal relations and spatial interactions.

- The 2nd modification: Introduce shortcut connections between the input and output of every LSTM layer, aiming to directly pass the prior information of historical trajectories to the subsequent layers. This structure can alleviate the gradient vanishment in the back-propagation [26].

Their model was trained and evaluated on the NGSIM I-80 and US-101 datasets. The obtained prediction results yield higher accuracy than the result of one the-state-of-the-art model (i.e., the M-LSTM in [9]), which proves the effectiveness of their modifications.

#### GISNet :Graph-Based Information Sharing Network For Vehicle Trajectory Prediction (Zhao et al. 2020)

Many papers have been published to improving the trajectory prediction algorithm [8] [9] [10]. Despite the efforts,  the accuracy of the existing prediction models is not high enough. This is because the surrounding vehicles will also respond to its environment and adjust its trajectory accordingly. Without considering this, the model can not make an accurate trajectory prediction. Hence it is necessary to have information from all neighboring vehicles, and consider the potential evolvements of their trajectories in the near future. For example, the human drivers observe and surmise other drivers latent intention from the mirrors of the car. To emulate this behavior, an information sharing network should be established among all vehicles.

target vehicle = the vehicle whose trajectory is to be predicted

neighbors = the surrounding vehicles.

They proposed a novel graph-based information sharing network (GISNet), which allows the target and neighbor vehicles to propagate and learn the trajectory features among themselves. Our proposed network is evaluated using the NGSIM highway vehicle trajectory dataset. Compared with other existing trajectory prediction methods, their approach can reduce the prediction error by up to 50.00%. The following summarizes the major contributions of our work:

- A new trajectory prediction model is developed which grants the information sharing among the graph neural network.

- The prediction is based on the embedding feature, which is derived from multi-dimensional input sequences including the historical trajectory of target and neighboring vehicles, and their relative social positions.

- The model allows us to consider the latent intention of surrounding neighbors during the prediction. Compared with other existing trajectory prediction methods, our approach can reduce the prediction error by up to 50.00% and achieve the state-of-the-art performance.

#### A Dual Learning Model for Vehicle Trajectory Prediction

In a reliable trajectory prediction system, accuracy is not the only indicator of system effectiveness. The horizon of prediction, ability to be generalised to new traffic environments and consideration of future vehicle trajectory as influenced by the surrounding vehicle trajectories (inter-vehicle interaction) are also significant.

Author proposed a Dual Learning Model (DLM) for vehicle trajectory prediction that identifies salient information

in an Occupancy Map (OM) and a Risk Map (RM) to learn about inter-vehicle interaction and associated risk within the traffic scene in an unsupervised manner. It is a data driven approach with two input channels, using a Long-Short-Term Memory (LSTM) encoder-decoder structure. This methodology is motivated by the success of LSTM networks in modelling non-linear temporal dependencies in sequence-to-sequence prediction. We have evaluated the proposedsystem with two distinct naturalistic highway driving scenarios which vary in their traffic flow and driving style to illustrate the ability of the proposed system to be extended for different environments.

The contributions of this study are:

- They inject the Risk Map into the trajectory prediction pipeline, where the proposed structure learns to automatically extract, store and retrieve salient information without any supervision.

- They test the proposed model on two distinct naturalistic highway driving datasets, containing more than 108570 unique trajectories from different datasets, to prove the independence of the proposed system from a specific environment.

- We extensively evaluate the performance of the proposed method with state-of-the-art methods. We show that our proposed method outperforms the state-of-the art methods.

They admit the limitation of considering other road environments such as curved roads or roundabouts at this stage of their research. The future direction of this research is to investigate the effectiveness of the proposed method for different scenarios such as roundabouts and intersections.

In addition, investigating the influence of Cooperative ITS (C-ITS), the effect of using communication information in the proposed dual learning Model for decision making in automated vehicles, is a worth future investigation.

## Long Short-Term Memory models and its alternatives

### What is a Long Short-Term Memory model?

#### Recurrent Neural Networks

Humans don't start their thinking from scratch every second. As you read this essay, you understand each word based on your understanding of previous words. You don't throw everything away and start thinking from scratch again. Your thoughts have persistence. Traditional neural networks can't do this, and it seems like a major shortcoming. For example, imagine you want to classify what kind of event is happening at every point in a movie. It's unclear how a traditional neural network could use its reasoning about previous events in the film to inform later ones. Recurrent neural networks address this issue. They are networks with loops in them, allowing information to persist.

A recurrent neural network can be thought of as multiple copies of the same network, each passing a message to a successor.

RNN's problem

Sometimes, we only need to look at recent information to perform the present task. For example, consider a language model trying to predict the next word based on the previous ones. If we are trying to predict the last word in "the clouds are in the sky," we don't need any further context – it's pretty obvious the next word is going to be sky. In such cases, where the gap between the relevant information and the place that it's needed is small, RNNs can learn to use the past information.

But there are also cases where we need more context. Consider trying to predict the last word in the text "I grew up in France… I speak fluent French." Recent information suggests that the next word is probably the name of a language, but if we want to narrow down which language, we need the context of France, from further back. It's entirely possible for the gap between the relevant information and the point where it is needed to become very large.

Unfortunately, as that gap grows, RNNs become unable to learn to connect the information. In theory, RNNs are absolutely capable of handling such "long-term dependencies." A human could carefully pick parameters for them to solve toy problems of this form. Sadly, in practice, RNNs don't seem to be able to learn them. The problem was explored in depth by Hochreiter (1991) [German] and Bengio, et al. (1994), who found some pretty fundamental reasons why it might be difficult.

#### Long Short-Term Memory networks

Long Short Term Memory networks – usually just called "LSTMs" – are a special kind of RNN, capable of learning long-term dependencies. Long Short-Term Memory (LSTM) is an RNN architecture specifically designed to address the vanishing gradient problem

They were introduced by Hochreiter & Schmidhuber (1997), and were refined and popularized by many people in following work.1 They work tremendously well on a large variety of problems, and are now widely used.

LSTMs are explicitly designed to avoid the long-term dependency problem. Remembering information for long periods of time is practically their default behavior, not something they struggle to learn!

The two technical problems overcome by LSTMs are vanishing gradients and exploding gradients, both related to how the network is trained.

### What are its limitations?

Danihelka et al. address two limitations of LSTM: (Danihelka et al. 2016)

- The first limitation is that the number of memory cells is linked to the size of the recurrent weight matrices. An LSTM with Nh memory cells requires a recurrent weight matrix with O(Nh2 ) weights.

- The second limitation is that LSTM is a poor candidate for learning to represent common data structures like arrays because it lacks a mechanism to index its memory while writing and reading.

About training RNN/LSTM: (Eugenio Culurciello 2018) RNN and LSTM are difficult to train because they require memory-bandwidth-bound computation, which is the worst nightmare for hardware designer and ultimately limits the applicability of neural networks solutions.

- In short, LSTM require 4 linear layer (MLP layer) per cell to run at and for each sequence time-step. Linear layers require large amounts of memory bandwidth to be computed, in fact they cannot use many compute unit often because the system has not enough memory bandwidth to feed the computational units. And it is easy to add more computational units, but hard to add more memory bandwidth (note enough lines on a chip, long wires from processors to memory, etc).

→ As a result, RNN/LSTM and variants are not a good match for hardware acceleration.

According to the website geeksforgeeks.org, LSTMs have a few drawbacks which are discussed as below:

- LSTMs became popular because they could solve the problem of vanishing gradients. But it turns out, they fail to remove it completely. The problem lies in the fact that the data still has to move from cell to cell for its evaluation. Moreover, the cell has become quite complex now with the additional features (such as forget gates) being brought into the picture.

- They require a lot of resources and time to get trained and become ready for real-world applications. In technical terms, they need high memory-bandwidth because of linear layers present in each cell which the system usually fails to provide for. Thus, hardware-wise, LSTMs become quite inefficient.

- With the rise of data mining, developers are looking for a model that can remember past information for a longer time than LSTMs. The source of inspiration for such kind of model is the human habit of dividing a given piece of information into small parts for easy remembrance.

- LSTMs get affected by different random weight initializations and hence behave quite similar to that of a feed-forward neural net. They prefer small weight initializations instead.

- LSTMs are prone to overfitting and it is difficult to apply the dropout algorithm to curb this issue. Dropout is a regularization method where input and recurrent connections to LSTM units are probabilistically excluded from activation and weight updates while training a network.

Sepp et al., the developers of LTSM, listed the limitation of their works as:

- The particularly efficient truncated backprop version of the LSTM algorithm will not easily solve problems similar to "strongly delayed XOR problems," where the goal is to compute the XOR of two widely separated inputs that previously occurred somewhere in a noisy sequence. The reason is that storing only one of the inputs will not help to reduce the expected error the task is non-decomposable in the sense that it is impossible to incrementally reduce the error by first solving an easier subgoal.

- Each memory cell block needs two additional units (input and output gate). In comparison to standard recurrent nets, however, this does not increase the number of weights by more than a factor of 9: each conventional hidden unit is replaced by at most 3 units in the LSTM architecture, increasing the number of weights by a factor of 32 in the fully connected case. Note, however, that our experiments use quite comparable weight numbers for the architectures of LSTM and competing approaches

- Generally speaking, due to its constant error flow through CECs within memory cells, LSTM runs into problems similar to those of feed-forward nets, seeing the entire input string at once. For instance, there are tasks that can be quickly solved by random weight guessing but not by the truncated LSTM algorithm with small weight initializations, such as the 500-step parity problem (see introduction to Section 5). Here, LSTM's problems are similar to the ones of a feed-forward net with 500 inputs, trying to solve 500-bit parity. Indeed LSTM typically behaves much like a feed-forward net trained by backprop that sees the entire input. But that's also precisely why it so clearly outperforms previous approaches on many non-trivial tasks with significant search spaces.

- LSTM does not have any problems with the notion of \recency" that go beyond those of other approaches. All gradient-based approaches, however, suffer from a practical inability to precisely count discrete time steps. If it makes a difference whether a certain signal occurred 99 or 100 steps ago, then an additional counting mechanism seems necessary. Easier tasks, however, such as one that only requires to make a difference between, say, 3 and 11 steps, do not pose any problems to LSTM. For instance, by generating an appropriate negative connection between memory cell output and input, LSTM can give more weight to recent inputs and learn decays where necessary.

Kewei Chen et al claimed that it is quite difficult for CPUs and GPUs to offer sufficient parallelism for the sequential components of LSTM model. (Chen et al. 2018) Taking account of computational efficiency, a special hardware architecture is necessary to implement LSTM in embedded systems. Most of the proposed LSTM hardware architectures are non-configurable, which implies they cannot support networks of different sizes and precision. Moreover, their resources consumption is too large for the requirement of the cost-sensitive Internet of Things (IoT).

### What are its alternatives?

#### Graph Neural Networks (GNN)

The first motivation of GNNs roots in convolutional neural networks (CNNs). CNNs can only operate on regular Euclidean data like images (2D grid) and text (1D sequence) while these data structures can be regarded as instances of graphs. Therefore, it is straightforward to think of finding the generalization of CNNs to graphs. As shown in Fig. 1, it is hard to define localized convolutional filters and pooling operators, which hinders the transformation of CNN from Euclidean domain to non-Euclidean domain.

→ GNN is suitable for graph data. But the dataset for VTP are the coordinates of vehicle and other numerical data.

→ GNN is better for trajectory extraction than trajectory prediction

#### GRU

A slightly more dramatic variation on the LSTM is the Gated Recurrent Unit, or GRU, introduced by Cho, et al. (2014). It combines the forget and input gates into a single "update gate." It also merges the cell state and hidden state, and makes some other changes. A gated recurrent unit (GRU) is basically an LSTM without an output gate, which therefore fully writes the contents from its memory cell to the larger net at each time step. The resulting model is simpler than standard LSTM models, and has been growing increasingly popular.

#### Recurrent Networks of Rectified Linear Units (Le, Jaitly, and Hinton 2015)

Recent research on deep feed-forward networks has also produced some impressive results [19, 3] and there is now a consensus that for deep networks, rectified linear units (ReLUs) are easier to train than the logistic or tanh units that were used for many years [27, 40]. At first sight, ReLUs seem inappropriate for RNNs because they can have very large outputs so they might be expected to be far more likely to explode than units that have bounded values. A second aim of this paper is to explore whether ReLUs can be made to work well in RNNs and whether the ease of optimizing them in feed-forward nets transfers to RNNs.

#### Independently Recurrent Neural Network (IndRNN) (Li et al. 2018)

Recurrent neural networks (RNNs) have been widely used for processing sequential data. However, RNNs are commonly difficult to train due to the well-known gradient vanishing and exploding problems and hard to learn long-term patterns. Long short-term memory (LSTM) and gated recurrent unit (GRU) were developed to address these problems, but the use of hyperbolic tangent and the sigmoid action functions results in gradient decay over layers. Consequently, construction of an efficiently trainable deep network is challenging. In addition, all the neurons in an RNN layer are entangled together and their behaviour is hard to interpret. To address these problems, a new type of RNN, referred to as independently recurrent neural network (IndRNN), is proposed in this paper, where neurons in the same layer are independent of each other and they are connected across layers. We have shown that an IndRNN can be easily regulated to prevent the gradient exploding and vanishing problems while allowing the network to learn long-term dependencies. Moreover, an IndRNN can work with non-saturated activation functions such as relu (rectified linear unit) and be still trained robustly. Multiple IndRNNs can be stacked to construct a network that is deeper than the existing RNNs. Experimental results have shown that the proposed IndRNN is able to process very long sequences (over 5000 time steps), can be used to construct very deep networks (21 layers used in the experiment) and still be trained robustly. Better performances have been achieved on various tasks by using IndRNNs compared with the traditional RNN and LSTM.

#### Temporal convolutional network (TCN)

#### Associative Long Short-Term Memory

## Methods to extract vehicle trajectory data from video

### Characteristics of available vehicle trajectory data sets

### A review of extraction techniques to collect vehicle trajectory

## Implementation of High-Performance Computing

# solutions

Three steps:

STEP 1:  Develop a new prediction algorithm for vehicle trajectories from highways and test it on a public dataset like NGSIM to show our improvement in performance

STEP 2:  Develop a new extraction algorithm for vehicle trajectories from traffic cameras. Compare and evaluate it with others

STEP 3: Develop a new prediction algorithm for vehicle trajectories from various types of roads and test it on the datasets from step 2.

If possible, each step can be written into a paper

# theory analysis

# implement steps

This is a 3.5-year project:

- First 9 months: Extraction of vehicle trajectory data from video

- 2 years: Vehicle trajectory prediction

- Last 9 months: Implementation of High-Performance Computing

Thank you for your comments on the later part. I will answer and rewrite those parts after finishing the previous sections.

## Task descriptions

### Extraction of vehicle trajectory data from video

- Where can we collect video footage? Free or Paid? Is the metadata of the video clip enough for our research?

- Which road segments are suitable for our research? We need to select at least one road for each road types

- How to apply R-CNN or similar techniques to extract vehicle trajectory from videos? Which one is the best for each road types

### Vehicle trajectory prediction

- How to apply LSTM RNN to predict vehicle trajectories? Can we develop a new novel model to carry out this task? If yes, is our model superior to the old ones?

### Implementation of High Performance Computing

- Which type of virtual machine is needed for our project?

- As our data is big, how to implement Big Data techniques in our model so that we can reduce the computation time and workload?→ reduce the cost

# evaluation plan

# research outcome prediction

Expectations when the project finishes:

- This project will help improve the movements and lane changing decisions of autonomous vehicles at any specific road types.

- New novel techniques for the extraction of vehicle trajectory data as well as the vehicle trajectory prediction are proposed.

- This project shows an example of extracting useful information from traffic cameras, whose primary functions are to measure traffic flow and to determine traffic light timing. Compared to the traditional way of collecting this kind of data, which uses a car that equipped with cameras and a GPS tracking device, our approach is much less expensive.

- With the help of Amazon EC2, autonomous vehicles can simulate the best trajectory at any road types in real-time

# Reference

Altché, Florent, and Arnaud de La Fortelle. 2018. “An LSTM Network for Highway Trajectory Prediction.” arXiv.org.

“Autonomous Vehicle Testing Permit Holders - California DMV.” https://www.dmv.ca.gov/portal/vehicle-industry-services/autonomous-vehicles/autonomous-vehicle-testing-permit-holders/ (July 8, 2020).

Chen, Kewei et al. 2018. “A Compact and Configurable Long Short-Term Memory Neural Network Hardware Architecture.” In Proceedings - International Conference on Image Processing, ICIP, IEEE Computer Society, 4168–72.

Cui, Jingze, Xian Zhou, Yanmin Zhu, and Yanyan Shen. 2017. “A Road-Aware Neural Network for Multi-Step Vehicle Trajectory Prediction.” In Database Systems for Advanced Applications, Springer International Publishing, 878–90. https://doi.org/10.1007/978-3-319-91452-7.

Dai, Shengzhe, Li Li, and Zhiheng Li. 2019. “Modeling Vehicle Interactions via Modified LSTM Models for Trajectory Prediction.” IEEE Access 7: 38287–96.

Danihelka, Ivo et al. 2016. “Associative Long Short-Term Memory.” http://arxiv.org/abs/1602.03032 (July 16, 2020).

Deo, Nachiket, and Mohan M. Trivedi. 2018a. “Convolutional Social Pooling for Vehicle Trajectory Prediction.” IEEE Computer Society Conference on Computer Vision and Pattern Recognition Workshops 2018-June: 1549–57.

———. 2018b. “Multi-Modal Trajectory Prediction of Surrounding Vehicles with Maneuver Based LSTMs.” In IEEE Intelligent Vehicles Symposium, Proceedings, Institute of Electrical and Electronics Engineers Inc., 1179–84.

Eugenio Culurciello. 2018. “The Fall of RNN / LSTM.” Towards data science. https://towardsdatascience.com/the-fall-of-rnn-lstm-2d1594c74ce0 (July 16, 2020).

KPMG. 2019. 2019 Autonomous Vehicles Readiness Index: Assessing Countries’ Preparedness for Autonomous Vehicles. https://assets.kpmg/content/dam/kpmg/xx/pdf/2019/02/2019-autonomous-vehicles-readiness-index.pdf.

Le, Quoc V., Navdeep Jaitly, and Geoffrey E. Hinton. 2015. “A Simple Way to Initialize Recurrent Networks of Rectified Linear Units.” http://arxiv.org/abs/1504.00941 (July 16, 2020).

Li, Shuai et al. 2018. “Independently Recurrent Neural Network (IndRNN): Building A Longer and Deeper RNN.” Proceedings of the IEEE Computer Society Conference on Computer Vision and Pattern Recognition: 5457–66. http://arxiv.org/abs/1803.04831 (July 16, 2020).

Matt Williams. 2014. “Permits for Self-Driving Cars a ‘Game-Changer,’ DMV Official Says.” https://www.govtech.com/state/Permits-for-Self-Driving-Cars-a-Game-Changer-DMV-Official-Says.html (July 8, 2020).

Messaoud, Kaouther, Itheri Yahiaoui, Anne Verroust-Blondet, and Fawzi Nashashibi. 2019. “Relational Recurrent Neural Networks For Vehicle Trajectory Prediction.” 2019 IEEE Intelligent Transportation Systems Conference, ITSC 2019 (October): 1813–18.

National Science and Technology Council, and US DOT. 2020. Ensuring American Leadership in Automated Vehicle Technologies, Automated Vehicles 4.0. https://www.transportation.gov/av/4.

Park, Seong Hyeon et al. 2018. “Sequence-to-Sequence Prediction of Vehicle Trajectory via LSTM Encoder-Decoder Architecture.” arXiv.org.

Patel, Sajan, Brent Griffin, Kristofer Kusano, and Jason J. Corso. 2018. “Predicting Future Lane Changes of Other Highway Vehicles Using RNN-Based Deep Models.” http://arxiv.org/abs/1801.04340 (July 8, 2020).

Xing, Yang, Chen Lv, and Dongpu Cao. 2020. “Personalized Vehicle Trajectory Prediction Based on Joint Time-Series Modeling for Connected Vehicles.” IEEE Transactions on Vehicular Technology 69(2): 1341–52.

Zhao, Ziyi, Haowen Fang, Zhao Jin, and Qinru Qiu. 2020. “GISNet: Graph-Based Information Sharing Network For Vehicle Trajectory Prediction.” arXiv. http://arxiv.org/abs/2003.11973.

Zhu, Jiacheng, Shenghao Qin, Wenshuo Wang, and Ding Zhao. 2019. “Probabilistic Trajectory Prediction for Autonomous Vehicles with Attentive Recurrent Neural Process.” arXiv (1). http://arxiv.org/abs/1910.08102.

Zyner, Alex, Stewart Worrall, and Eduardo Nebot. 2018. “Naturalistic Driver Intention and Path Prediction Using Recurrent Neural Networks.” IEEE Transactions on Intelligent Transportation Systems 21(4): 1584–94.