# Trajectory Planning and Tracking for Autonomous Overtaking: State-of-the-Art and Future Prospects

Shilp Dixit et al.

# Introduction

Modern cars = sensors + electronic systems → assisting ABS, cruise control, navigator.

Intelligent vehicles  = automated maneuvering (piloting) in various driving scenarios.

Overtaking - a complex driving task: both lateral and longitudinal motions of an overtaking vehicle (subject vehicle) while avoiding collisions with a slower moving vehicle (lead vehicle).

Others difficulties: 	environmental conditions like weather, visibility +

Road-users like small cars, buses, truck

Three typical overtaking maneuvering steps:

- lane change to overtaking lane,

- pass lead vehicle(s),

- lane change back to the original lane

Two types of lane change:

- Discretionary Lane Change (DLC): a driver desires a faster speed, greater following distance, a further line of sight, better ride quality, etc

- Mandatory Lane Change (MLC): follow the specific route due to compulsion arising from traffic rules

→ Overtaking maneuver = a complex task of dynamically choosing the best driving lane based on (i) legislation, (ii) driving intentions, and (iii) instantaneous traffic situation

For an autonomous vehicle, safety evaluation = subject vehicle’s states + surrounding information → tactical decisions.

Techniques for decision making:

- multi-level decision trees

- probabilistic weighted comparison of concurrent goals

- higher award seeking Markovian Decision Process algorithms

Four typical overtaking maneuvers:

- Standard: The subject vehicle approaches the lead vehicle and waits for a suitable opportunity to perform the maneuver

- Flying: The subject vehicle does not adjust its longitudinal velocity and is directly able to overtake the lead vehicle

- Piggy backing: The subject vehicle follows a preceding car as they both pass the lead vehicle

- 2+: The subject vehicle overtakes two or more lead vehicles in a single manoeuvre

A modern vehicle uses a host of on-board sensors to discern the environment, and Fig.2 illustrates the placement of an on-board sensor suite used to perform this task. The information from these sensors is combined and used for following missions:

- classify objects,

- track stationery and moving obstacles,

- identify safe driving zones, etc.

## Vehicle-to-everything (V2X) – a new rising communication technology

U.S. Department of Transportation views that V2X technologies have the potential for significant transportation safety and mobility benefits, both on their own and as complementary technologies, when combined with in-vehicle sensors supporting the integration of automated vehicles and other innovative applications. The U.S. NHTSA estimated a minimum of 13% reduction in traffic accidents if a V2V system were implemented, resulting in 439,000 fewer crashes per year.

V2X = V2I (vehicle-to-infrastructure), V2N (vehicle-to-network), V2V (vehicle-to-vehicle), V2P (vehicle-to-pedestrian), V2D (vehicle-to-device) and V2G (vehicle-to-grid).

Public data for V2X: http://vehicle2x.net/v2x-measurements/

In real situations, not all traffic participants are connected and autonomous

Different methods for motion planning for intelligent autonomous vehicles based on abstraction levels of traffic motion are classified as

- Physics-based

- Maneuver-based

- Interaction-aware

# System architecture

An autonomous vehicle utilizes information from on-board sensors (Radar, LiDAR, camera, etc.) and o
-board information via V2X communications to generate a real-time environmental representation

# Trajectory planning

An autonomous vehicle relies on real-time vehicle state and environment:

- to derive a local trajectory that ensures a safe passage

- to minimize the deviation from the overall journey trajectory (global trajectory).

Local trajectory planning = “real-time planning of the vehicle’s transition from one feasible state to the next while satisfying the vehicle’s kinematic limits based on vehicle dynamics and constrained by occupant comfort, lane boundaries, and traffic rules, while, at the same time, avoiding obstacles.”

Five well known techniques:

| Control strategy | Strength | Weakness |
| --- | --- | --- |
| Potential fields | Optimality of searched path guaranteed Collision free path guaranteed | High computation cost Inability to handle system constraints No systematic procedure to consider environmental uncertainties |
| Cell Decomposi- tion | Guaranteed collision-free trajectories | Computation requirements sensitive to traffic density Computed paths are jerky No systematic procedure to consider environmental uncertainties |
| Interdisciplinary Techniques | Reduced complexity of collision avoidance as trajectory planning converted to reference tracking problem Real-time capable | Experimentally unproven No systematic design procedure Do not consider uncertainties in environment perception while generating reference points |
| Optimal Control | Generate collision-free trajectories Ability to include kinematic constraints | Unsuitable for high-speed driving maneuvers with large angles of tire slip Inability to consider tire dynamics |
| Model Predictive Control (MPC) | Include vehicle and tire dynamics Systematic handling of constraints and traffic uncertainties Computational requirements independent of the environment | Optimization sensitive to many constraints Computation complexity scales quickly with high-order system models, non-linearity, and non- convexity of constraints |

# Trajectory tracking

Vehicle trajectory tracking (lateral-longitudinal control) is a mature scientific field.

Useful properties for assessing tracking controllers for autonomous vehicle applications:

- Real-time capability: The control law needs to be implementable on a vehicle’s Electronic Control Unit (ECU) and function within the calculation time

- Robustness: The designed controller should be robust against system non-linearities, model parameter variations, and external disturbances

- Operating Range: The tracking controller should ideally work across the entire range of vehicle speeds (0-120 km/h)

- Controller parameter tuning: A systematic tuning procedure for the controller parameters allows for a structured controller design procedure

More: Modelling and Control Strategies in Path Tracking Control for Autonomous Ground Vehicles : A Review of State of the Art and Challenges

| Control Strategy | Strength(s) | Weakness |
| --- | --- | --- |
| Geometric & Kinematic | Adequate performance (experimentally validated) in conditions without disturbances (e.g., wind, road banking) Good tracking performance and robustness at moderate speeds (e.g., kinematic) | Do not consider vehicle dynamics Steady-state error increases for high-speed driving (e.g., geometric) Unsuitable for high-speed driving as dynamics are neglected (e.g., kinematic) Requires smooth and continuous reference trajectories |
| Classical | Established method with good performance for non-linear systems Robust closed-loop performance against uncertainties and noise (e.g., SMC) | Tuning of controller parameters is tricky (e.g., PID) Robust performance only in limited scenarios (e.g., SMC) Control law is sensitive to path curvature variations (e.g., SMC) |
| Dynamic state feedback | Consider vehicle dynamics in calculating control law Optimization shifted offline resulting in simple implementation of control law | Obtaining vehicle states (e.g., wheel forces, slip angles, torques etc.) is non-trivial Control law is sensitive to path curvature variations (e.g., LQR) |
| Neural Network | Sufficient training can make the behavior very human-like to make the automated car feel natural | Controller tuning requires simulation with large amounts of real world (training) data No failure explanations possible |
| Fuzzy Logic | Closed-loop system acts similar to a human-driver (because of human-like rules) | Controller tuning is not systematic with no formal stability analysis Rules can become unmanageable if number of variables is large |
| Model Predictive Control (MPC) | Systematic design procedure Ability to include system and actuator constraints in design procedure Inclusion of vehicle and tire dynamics in control problem | Non-linear MPCs with have high computing requirements making them unsuitable for high-speed driving environments The tracking performance is sensitive to the accuracy of prediction model Larger tuning parameter set compared to industry standard PID |

# Modelling and Control Strategies in 
Path Tracking Control for 
Autonomous Ground Vehicles: 
A Review of State of the Art and Challenges

Noor Hafizah Amer

# Introduction

For an automated vehicle system, level of autonomy may vary according to six different levels as outlined by SAE-J3016 [4].

• Level 0: Vehicle control lies solely on human operator. Automated system may issue warnings.

• Level 1: Several assistance may be available automatically such as Adaptive Cruise Control (ACC), Parking Assistance with automated steering, and Lane Keeping Assistance (LKA).

• Level 2: The automated vehicle system executes accelerating, braking, and steering. The automated system can deactivate immediately upon takeover by the driver.

• Level 3: The automated vehicle may perform auto-pilot within known, limited environments (such as freeways).

• Level 4: The automated vehicle system can control the vehicle in all direction throughout all but a few environments such as severe weather. The driver must enable the automated system only when it is safe to do so. When enabled, driver attention is not required.

• Level 5: The automated vehicle system has full autonomy with no human intervention required. The autonomous can drive itself to any location where it is legal to be operated.

There are three basic stages and modules in the system which are:

• Sensing and Perception – To provide real time data to let the system know the real time location and environment around the vehicle and prepare the raw data in format that is feasible for the system to process.

• Planning – To use the data provided by Sensing & Perception to dictate the safe and feasible path for the vehicle to follow.

• Control – Contain control strategies to move vehicle to the desired path. This include actuator control of each sub-system.

# Vehicle Model Used in Path Tracking Control

## Geometric Vehicle Model

Ackerman steering configurations: the line perpendicular to each of the vehicle wheel should intersect at the midway point of the vehicle cornering arc where the radius of turn is R.

## Kinematic Vehicle Model

Kinematic vehicle model = describing the motion of the vehicle in terms of its position, velocity and acceleration without any regard of its internal dynamics.

## Dynamic Vehicle Model

Dynamic vehicle model = describing the motion of the vehicle in terms of its position, velocity and acceleration

by considering the internal forces, energy or momentum within the system

## Summary on Vehicle Models

Vehicle modelling is particularly an important first step used to (a) simulate the vehicle responses and (b) derive

a controller algorithm.

In all categories, vehicle modelling was done in two ways, full vehicle as well as the half vehicle model also known as bicycle model.

Many notable software that is capable to predict and simulate the overall vehicle response such as CarSim and MSC ADAMS

# Trajectory Control Strategies

Trajectory tracking control in autonomous vehicles is aimed to provide sufficient steering input + throttle and braking input to control the direction and speed of the vehicle → guide the controlled vehicle along a pre-defined path.

This review particularly will cover the control strategy used in order to control the steering angle, δ and vehicle speed v along the pre-defined path.

The overall trajectory tracking control = three motion tasks. A suitable controller can be developed by solving these

three tasks together:

- Point-to-point motion: treat the whole motion as a basic task of reaching a defined goal point from an initial point of starting, regardless the path and trajectory. concern on the convergence and stabilization problem to a final position value

- Path following: the path connecting the starting point and goal point in such that the task is to reach and follow the given geometric path. Require the vehicle to be guided following the path, concerning the steering control of the vehicle

- Trajectory tracking: concerns on the task for the vehicle to reach and follow the given geometric path within an associated timing law = concerns on the time taken for the vehicle to complete the given path, concerning the speed control of the vehicle.

## Geometric and Kinematic Controller

### Follow the Carrot

### Pure Pursuit

### Stanley Method

## Dynamic Controllers

Dynamic controllers in path tracking control include the dynamic properties of the vehicles in their control law.

## Optimal Controller

LQR is one of the most popular optimal control theories where the controller gain was determined using linear quadratic optimisation approach.

Standard method to derive the control law can be found in textbooks:

- Rajamani, R.: Vehicle Dynamics and Control, 2 ed. Mechanical Engineering Series (2012)

- Tewari, A.: MODER CONTROL DESIGN (2002)

## Adaptive and Intelligent Controller

Adaptive controllers are usually developed for studies or applications that require high robustness to certain disturbance and changes.

Neural networks and fuzzy sets can be seen used in a lot of adaptive and intelligent control due to its ability in guiding the controller to make fast decisions. Implementations of decision-making algorithm using these two approaches can be categorised into two, namely supervisory or controller type implementation.

## Model Based Controllers

Model based controllers are sometimes regarded as adaptive controller. While the claim is valid, especially for controllers that employed model based predictions to estimate the ideal input to adapt to any changing vehicle conditions, model based controller can be regarded as one whole class.

Model predictive controller (MPC) usually used a linear or nonlinear plant model to predict the required control input for the plant. This method usually involves optimization procedure in order to get the optimal value for the plant input.

The most popular model predictive control (MPC) approach was Ollero, Amidi which proposed a generalized predictive controller that was intended to minimize a cost index containing errors between predicted steering angle and the desired steering angle.

## Classical Controllers

The classifications of classical controllers are applied to PID controllers and sliding mode controller (SMC).

PID controller is a common controller usually used in industrial applications due to its simple design and theory. It consists of three terms and usually triggered by the error between actual response and a desired response. The three terms are Proportional, P, Integral, I and Derivative, D, which corresponds to the action that each term applied to the triggering error, respectively. In trajectory tracking, it usually used to control the steering input as well as velocity input depending on a desired steering and velocity received from trajectory planner.

Sliding mode controller (SMC) is another classical controller. In SMC, the state feedbacks as well as the control signals are treated as discontinuous functions which make it unaffected to parametric uncertainties and external disturbances. → suitable for nonlinear systems. The name “Sliding Mode” = the motion of the system as it slides along the boundaries (called sliding surfaces) of the control structure. The control law utilizes fast switching strategy to drive and maintain the system’s state trajectory towards the chosen sliding surfaces as depicted in Fig. 14. With this feature, SMC offers a nonlinear controller with fast response and excellent robustness under system uncertainties and external disturbances.

# Performance Criteria for Path Tracking Controller

# Research Challenges in Path Tracking Control

- Vehicle Model

- Controller Development

  - Route/Path Definition

  - Robustness of controller to navigate various paths.

  - Adaptive Controllers

- Implementation of Path Tracking Control

  - Sensing and Perception

  - Testing

  - Performance Criteria

Overall challenges in Path Tracking Control

Overall, path tracking control focus on guiding the vehicle on a pre-defined path usually given by the on-board path planner or motion planner.

Main challenge in this field = the ability of the controller to navigate different type of road curvature, with different speed and different road conditions.

Integrated control should be developed in order to combine the steering, throttle/braking, and suspension controls for coordinated control in lateral, longitudinal and vertical direction while navigating through various road conditions.

In terms of lateral and longitudinal controls, an adaptive solution may offer good progress with a basic geometric controller. This will combine the simplicity of the geometric controller with the robustness aspect that can be offered by an adaptive controller approach.